- AM

clc;

clear all;

fs=80000;

fm=3000;

fc=30000;

Ac=1;

Am=1;

ka=1;

t=0:0.1/fs:0.001;

m=Am.*cos(2*pi*fm*t);

subplot(311);

plot(t,m);

title('Message signal');

xlabel('Time');

ylabel('Amplitude');

c=Ac.*cos(2*pi*fc*t);

subplot(312);

plot(t,c);

s=(1+ka.*m).*c;

subplot(313);

plot(t,s);

hold on

plot(t,1+ka*m,'r--');

plot(t,-1-ka*m,'r--');

hold off

r1= s.*c;

[b a] = butter(3,0.01);

mr1= filter(b,a,r1);

% subplot(414);

figure

axis([0 0.001 0 2]);

plot(t,3*mr1);

- DSBSC

fs=100000;

fm=6*10^3;

fc=50*10^3;

Am=1;

Ac=1;

t=0:0.1/fs:0.001;

m=Am*cos(2*pi*fm*t);

subplot(411)

plot(t,m);

c=cos(2*pi*fc*t);

subplot(412);

plot(t,c);

s=c.*m;

subplot(413);

hold on

plot(t,m,'k--');

plot(t,-m,'k--');

plot(t,s);

r1= s.*c;

[b a] = butter(3,0.01);

mr1= filter(b,a,r1);

subplot(414);

axis([0 0.001 0 2]);

plot(t,3*mr1);

- FM

clear all

fs=80*10^3;

fm=6*10^3;

fc=50*10^3;

Am=5;

Ac=5;

B=5;

t=0:0.1/fs:0.001;

m=Am*cos(2*pi*fm*t);

subplot(311)

plot(t,m);

c=cos(2*pi*fc*t);

subplot(312);

plot(t,c);

s=Ac.*cos(2*pi*fc*t + B.*sin(2*pi*fm*t));

subplot(313);

hold on

plot(t,m,'k--');

plot(t,s);

- PAM

clear all;

clc;

fs = 10000;

fc = 100;

fm =4;

t=1;

n = [0:1/fs:t];

n = n(1:end-1);

m = sin(2*pi*fm*n);

subplot(311);

%plot(n,m);

dutycycle = 50;

s = square(2*pi*fc*n,dutycycle);

plot(s);

s(find(s<0))=0;   %to make it unipolar

subplot(312)

plot(n,s);

ylim([0 1.2]);

%l=length(n);

period_sam = length(n)/fc;     %to find the number of samples in one period

ind = 1:period_sam:length(n);   %to find the starting sample index

on_samp = ceil(period_sam * dutycycle/100);   %no. of samples in on period of time

pam = zeros(1,length(n));

for i =1:length(ind)

pam(ind(i):ind(i)+on_samp) = m(ind(i));

end

subplot(3,1,3);

plot(n,pam);

hold on

plot(n,m,'r--');

hold off

%ylim([-1.5 1.5]);

- PPM

clc;

clear all;

t = 0:0.001:1;

fc = input('Enter the Frequency of Carrier Signal (Sawtooth) = ');

fm = input('Enter the Frequency of Message Signal (Sinusoidal) = ');

a = input('Enter the Amplitude of Carrier Signal = ');

b = input('Enter the Amplitude of Message Signal(should be < Carrier) = ');

vc = a.*sawtooth(2*pi*fc*t);

vm = b.*sin(2*pi*fm*t);

n = length(vc);

for i = 1:n

if (vm(i)>=vc(i))

pwm(i) = 1;

else

pwm(i) = 0;

end

end

% Representation of the Message Signal

subplot(4,1,1);

plot(t,vm,'black');

xlabel('Time ----->');

ylabel('Amplitude ----->');

title('Message Signal');

legend('Message Signal ---->');

grid on;

% Representation of the Carrier Signal

subplot(4,1,2);

plot(t,vc);

xlabel('Sample ----->');

ylabel('Amplitude ----->');

title('Carrier Signal');

legend('Carrier Signal ---->');

grid on;

% Representation of the PWM Signal

subplot(4,1,3);

plot(t,pwm,'red');

xlabel('Sample ----->');

ylabel('Amplitude ----->');

title('PWM Signal');

legend('PWM Signal ---->');

axis([0 1 0 2]);

grid on;

Pulsel = (pwm <1);

Temp  = [ 0 Pulsel];

Pulse2 = Pulsel - Temp(1:end-1);

Pulse2(Pulse2<0) = 0;

subplot(4,1,4);

plot(t,Pulse2,'green');

xlabel('Sample ----->');

ylabel('Amplitude ----->');

title('PPM Signal');

legend('PPM Signal ---->')

- PWM

clc;

clear all;

t = 0:0.001:1;

fc = input('Enter the Frequency of Carrier Signal (no of cycles in Sawtooth signal) = ');

fm = input('Enter the Frequency of Message Signal (no of cycles in Sinusoidal signal) = ');

a = input('Enter the Amplitude of Carrier Signal = ');

b = input('Enter the Amplitude of Message Signal(should be < Carrier amp) = ');

c = a.*sawtooth(2*pi*fc*t);

m = b.*sin(2*pi*fm*t);

n = length(c);

for i = 1:n

if (m(i)>=c(i))

pwm(i) = 1;

else

pwm(i) = 0;

end

end

% Representation of the Message Signal

subplot(3,1,1);

plot(t,m,'black');

xlabel('Time ----->');

ylabel('Amplitude ----->');

title('Message Signal');

legend('Message Signal ---->');

grid on;

% Representation of the Carrier Signal

subplot(3,1,2);

plot(t,c);

xlabel('Sample ----->');

ylabel('Amplitude ----->');

title('Carrier Signal');

legend('Carrier Signal ---->');

grid on;

% Representation of the PWM Signal

subplot(3,1,3);

plot(t,pwm,'red');

xlabel('Sample ----->');

ylabel('Amplitude ----->');

title('PWM Signal');

legend('PWM Signal ---->');

axis([0 1 0 2]);

grid on;

- SSBSC

fs=100000;

fm=3*10^3;

fc=30*10^3;

Am=1;

Ac=1;

t=0:0.1/fs:0.001;

m0=Am*cos(2*pi*fm*t);

m90=Am*sin(2*pi*fm*t);

subplot(411)

plot(t,m0);

hold on

plot(t,m90,'r');

hold off

c0=cos(2*pi*fc*t);

c90=sin(2*pi*fc*t);

subplot(412);

plot(t,c0);

hold on

plot(t,c90,'r');

hold off

s0=c0.*m0;

subplot(413);

plot(t,s0);

hold on

plot(t,m0,'k--');

plot(t,-m0,'k--');

plot(t,s0);

s90=c90.*m90;

subplot(414);

plot(t,s90);

hold on

plot(t,m90,'k--');

plot(t,-m90,'k--');

plot(t,s90);

figure

subplot(311)

plot(t,s0+s90);

%hold on

%plot(t,s0-s90,'r');

%hold off